Electrical Systems National, Domestic, and IoT Device

Edirimanna Y.H.

E/20/089

1. Sri Lankan Electricity System

1.1 How Power Is Generated and Transmitted to Houses

Sri Lanka uses mainly 3 methods:

- Hydropower (Mahaweli reservoirs and other major dams)

- Thermal power (coal at Norochcholai, diesel and furnace oil plants)

- Renewables (solar farms, rooftop solar, wind)

Step-by-step flow:

- Generation
Power is produced at power plants (11 kV–15 kV for most hydros, much higher for thermal stations).

- Step-up transformation
Voltage is increased using step-up transformers to 132 kV, 220 kV, or 400 kV for efficient long-distance transmission.

- Transmission
High-voltage AC lines carry energy across the country via the CEB national transmission grid.

- Step-down at Grid Substations
Voltage is reduced to 33 kV or 11 kV at primary substations.

- Distribution
Local substations further reduce voltage to 400/230 V for homes.

- Supply to Houses
Final delivery uses service drops, meters, MCBs, and consumer wiring.

1.2 Voltages Used

- 400 V (three-phase) → Line-to-line

- 230 V (single-phase) → Line-to-neutral

- 50 Hz frequency (standard)

- High-voltage lines: 132 kV, 220 kV, and sometimes 400 kV

2. Electrical System in a House

2.1 Wiring

A typical house uses:

- Single-phase 230 V supply

- Main components:

  - Service cable

  - Energy meter

  - Main breaker (MCB)

  - Residual Current Device (RCD) / RCCB

  - Distribution board with separate circuits

Circuit types:

- Lighting circuits (1–1.5 mm² wires)

- Socket/outlet circuits (2.5 mm² wires)

- High-load appliances (air conditioners, heaters, ovens) may use dedicated breakers and 4–6 mm² cables

Color coding :

- Live – Brown

- Neutral – Blue

- Earth – Green/Yellow

2.2 Protection

Protection ensures safety against fire and electrocution:

- Main MCB – protects against short circuits/overload

- RCD / RCCB – protects against electric shock by monitoring leakage currents

- Earthing – diverts fault current safely into the ground

- Surge protectors – protect against lightning and grid surges

3. Electrical System for an IoT Device

3.1 Wiring

- Clear power flow from AC input through all components

- Ground connections for safety

- DC distribution bus to power all IoT components

3.2 Protection

IoT devices are sensitive, so basic protection is essential:

- Fuse– prevents overcurrent

- TVS diode – protects against voltage spikes

- Reverse-polarity protection – using a diode or MOSFET

- Voltage regulator – ensures stable voltage

- ESD protection – for sensors and communication pins

3.3. Backup Power:

- Battery system with charge controller

- Automatic switchover when main power fails

- Keeps the IoT device running during power outages